# Experiment 7:  Stegnography

| Roll No. A166 | Name: Vedashree Bhandigare |
| --- | --- |
| Class : B.Tech CE Div-B | Batch: B2 |
| Date of Experiment: 30/08/2025 | Date of Submission |

Aim: To recover email messages from packet capture and analyze the same.

Learning Outcomes:

After completion of this experiment, student should be able to analyze SMTP packets using wireshark.

Case:

After being released on bail, Ann Dercover disappears! Fortunately, investigators were carefully monitoring her network activity before she skipped town. “We believe Ann may have communicated with her secret lover, Mr. X, before she left,” says the police chief. “The packet capture may contain clues to her whereabouts.”

You are the forensic investigator. Analyze the given packet capture to answer following questions.

- Provide any online aliases or addresses and corresponding account credentials that may be used by the suspects under investigation.

- Who did Ann communicate with? Provide a list of email addresses and any other identifying information.

- Extract any transcripts of Ann’s conversations and present them to investigators

- If Ann transferred or received any files of interest, recover them

- Are there any indications of Ann’s physical whereabouts? If so, provide supporting evidence.

Network:

- Internal network:192.168.30.0/24

- DMZ: 10.30.30.0/24

- The Internet: 172.30.1.0/24 (for purpose of this case study)

Evidence:

Investigators provide you with a packet capture from Ann’s home network. Evidence file is available on local server under ISAF\AnnCase.They also inform you that in the course of their monitoring, they have found that Ann’s laptop has a MAC address of 00:21:70:4D;4F:AE.

Procedure

- Open evidence file using wireshark.

- Click statistics-àProtocol Hierarchy. Note the percentage of following protocols

IP, UDP, TCP, DNS, bootstrap, SMTP, IMAP and HTTP.

- Use wireshark display filter eth.addr==00:21:70:4d:4f:ae and bootp. Note MAC address, IP address, Host Name, lease time, DNS server address, Router Address, subnet mask, etc.

- Use wireshark display filter smtp.command_line. Note number of EHLO messages, email addresses.

- Use wireshark display filter smtp. Right click on first message and follow TCP stream. Is authentication detail encrypted?

- Note username and password. Decode using base 64. Note the content of email.

- Using networkminer software (available on local host), analyze given packet capture and note the following details related to email.

  - To and from email ids

  - Subject

  - Message ids

  - Timestamp values

  - Attachment if any

- Recover attachment and note the hash values of the same.

- Upload your document on Student Portal by answering questions given at the end of case description.

Review questions:

1. Develop timeline for the given case.

Ans :- To create a timeline, you’ll need to analyze the packet capture file (which should be in .pcap format) using Wireshark or any packet analysis tool. Here’s how you can proceed:

Step 1: Open the packet capture file in Wireshark.

Step 2: Filter traffic by SMTP protocol to narrow down the packet list to only those relevant to email communications. You can use the filter smtp to find relevant packets.

Step 3: Look for packets that are SMTP requests (such as HELO, MAIL FROM, RCPT TO, and DATA) to identify the interactions that involve email transmissions.

Step 4: Analyze timestamps for each packet to build the timeline. Check when Ann’s email communications started and ended, along with any other significant network activity (e.g., file transfers, login attempts).

Step 5: Look for session durations, IP addresses, and MAC addresses associated with her laptop. You may correlate these timestamps with network activity (e.g., DNS lookups, email exchanges) to piece together a timeline of events leading to Ann's disappearance.

| Example Timeline Format: Date/Time | Event Description |
| --- | --- |
| 2025-03-05 10:05 AM | Ann logs into email server using IP: 192.168.30.10 |
| 2025-03-05 10:15 AM | First SMTP communication sent to xyz@abc.com |
| 2025-03-05 10:30 AM | Files transferred to external address |
| 2025-03-05 10:45 AM | Last email sent before Ann's disappearance |

2. Who did Ann communicate with? Provide a list of email addresses and any other identifying information

Ans :-

Step 1: In the packet capture, look for SMTP MAIL FROM, RCPT TO, and DATA packets. These will contain the sender and recipient email addresses.

  - Step 2: Focus on identifying sender and receiver email addresses, which will be in the SMTP commands:

  - MAIL FROM: <sender_email>

  - RCPT TO: <recipient_email>

  - Step 3: Look for patterns or repeated interactions with specific email addresses that could point to Ann’s communication with key people (such as the secret lover, Mr. X).

  - Step 4: Examine any additional packets where login information might be included (such as from SMTP AUTH commands) to gather any associated usernames and possibly passwords (if the protocol was unencrypted or weakly encrypted).

Example Email List:

• Ann’s Email: ann.dercover@example.com (from MAIL FROM)

• Mr. X’s Email: mr.x.secret@example.com (from RCPT TO)

  - • Others:

  - recipient1@abc.com

  - recipient2@xyz.com

If credentials like username/password appear in unencrypted traffic (e.g., in the SMTP AUTH command), record those details.

3. Are there any indications of Ann’s physical whereabouts? If so, provide supporting evidence.

Ans :- To determine Ann’s physical location, follow these steps:

Step 1: Look at the IP addresses involved in the communication. If Ann was at home, she would likely be using the internal network (IP range: 192.168.30.0/24).

Step 2: Analyze any external IPs Ann may have connected to (in the Internet range: 172.30.1.0/24). Use reverse IP lookups or geolocation tools to identify where these IPs are located.

Step 3: Check for DNS queries that might reveal URLs or locations tied to email services or online activity, which may provide clues to her location.

Step 4: If the packet capture includes DHCP logs or ARP requests, use the MAC address (00:21:70:4D:4F:AE) to verify when Ann’s device was active on the network and cross-check with any time zones of the external services she was communicating with.

Example Evidence:

• Internal IP: 192.168.30.10 (likely her device)

• External IP: 172.30.1.25 (corresponding to a specific region or city)

• Geolocation: IP 172.30.1.25 resolved to a location near New York City.

OUTPUT